VAN ALLEN BELTINATOR

IGLOO Deployment Manual

Panoramic Equirectangular Rendering  |  Split-Mode Tablet Control

March 2026  |  Prepared for NOAA Space Weather Operations

Dr. Nate Dailey  |  The MITRE Corporation  |  SpaceSherpa IR&D

# 1. System Overview

The Van Allen Beltinator IGLOO deployment uses a split-mode architecture that separates the visualization renderer from the user control interface. The 3D radiation belt scene renders on the IGLOO projection computer, where a GPU-accelerated GLSL shader converts a cubemap capture of the scene into a continuously-updating equirectangular panoramic image (2:1 aspect ratio). This panorama is displayed in a dedicated output window that the IGLOO Core Engine captures as a video source for reprojection onto its six projection surfaces.

A Surface tablet connected via WebSocket provides the full desktop control panel, allowing operators to adjust physics parameters, switch storm presets, navigate the camera, and focus on satellites — all wirelessly relayed to the IGLOO renderer in real time. A Web Worker-based background rendering system ensures the panoramic output continues at full frame rate even when the browser window loses focus.

# 2. Architecture

## 2.1 Components

| Component | Machine | Role |
| --- | --- | --- |
| Relay Server | Any PC on LAN | Node.js (Express + ws). Routes WebSocket messages between controller and renderer. Serves HTML over HTTPS on port 8443. |
| IGLOO Renderer | IGLOO projection PC | Opens ?mode=igloo in Chrome. Hides all UI. Auto-enables panoramic rendering. Produces equirectangular output. Receives state/camera/preset commands from relay. |
| Tablet Controller | Surface tablet | Opens ?mode=controller in Chrome. Shows full desktop UI with sliders, presets, computed values, equations, and camera control pad. Sends all input to relay. |

## 2.2 Data Flow

Tablet → Relay → IGLOO: State changes (all slider values), preset selection (quiet/moderate/intense/recovery), camera orientation (camH, camV, camDist), satellite focus commands, zoom-back commands.

IGLOO → Relay → Tablet: Computed values (bounce factor, PSD, flux, D_LL, Kp, magnetopause, loss cones, trapping status) updated every second for synchronized display.

## 2.3 Panoramic Output Pipeline

The panoramic renderer works in three GPU stages per frame:

Stage 1: A THREE.CubeCamera renders the scene into a cubemap (six faces, configurable resolution from 512² to 2048² per face).

Stage 2: A custom GLSL fragment shader converts the cubemap to an equirectangular projection by mapping each output pixel’s UV to longitude/latitude, then to a 3D direction vector for cubemap sampling. This runs entirely on the GPU as a fullscreen quad render pass.

Stage 3: The equirectangular image is read back from the GPU render target and drawn to a canvas in a dedicated popup window. The IGLOO Core Engine captures this window as a video source.

# 3. Prerequisites

## 3.1 Software

•  Node.js v18+ with npm

•  npm packages: ws, express

•  OpenSSL (for SSL certificate generation)

•  Google Chrome (latest stable) on IGLOO PC and tablet

## 3.2 Hardware

•  IGLOO PC with GPU capable of 6× cubemap rendering at target resolution (RTX 3070+ recommended)

•  Surface tablet or any device with modern browser and touchscreen

•  Local network connecting all machines

## 3.3 Network

•  Port 8443 (HTTPS/WSS) open between all machines

•  All machines on the same LAN

# 4. File Inventory

| File | Description |
| --- | --- |
| beltinator_vr.html | Complete single-file application. Contains Three.js visualization, physics engine, desktop UI, WebXR VR support, panoramic rendering with GLSL shader, WebSocket client, and background rendering worker. Rename to index.html for deployment. |
| beltinator_relay.js | Node.js WebSocket relay server. Serves HTML over HTTPS, bridges messages between controller and IGLOO. Requires ws and express npm packages. |
| key.pem / cert.pem | SSL certificates (generated during setup). Required for HTTPS. |

# 5. Setup Procedure

## Step 1: Create Project Directory

mkdir beltinator-igloo && cd beltinator-igloo

npm init -y && npm install ws express

## Step 2: Copy Files

Place beltinator_vr.html and beltinator_relay.js in the directory. Optionally rename the HTML:

cp beltinator_vr.html index.html

NOTE: The relay server checks for index.html first, then beltinator_vr.html. Either name works.

## Step 3: Generate SSL Certificates

openssl req -x509 -newkey rsa:2048 -keyout key.pem -out cert.pem -days 365 -nodes

Enter the relay server’s IP address when prompted for Common Name (CN).

IMPORTANT: Both the IGLOO browser and tablet browser will show a security warning for self-signed certificates. Click Advanced → Proceed on each browser once.

## Step 4: Find Server IP Address

Windows:   ipconfig

Mac/Linux: ip addr

Note the LAN IP (typically 192.168.x.x or 10.x.x.x).

## Step 5: Open Firewall

Windows: netsh advfirewall firewall add rule name="Beltinator" dir=in action=allow protocol=TCP localport=8443

Linux:   sudo ufw allow 8443/tcp

# 6. Running the System

## 6.1 Startup Sequence

1.  Start the relay server:

node beltinator_relay.js

Verify the startup banner shows IGLOO, Controller, and Standalone URLs.

2.  Open the IGLOO renderer on the projection PC:

https://<relay-ip>:8443/?mode=igloo

Accept the SSL warning. Verify "igloo connected" in the relay console. The page will auto-hide all UI panels and auto-enable panoramic rendering after 1 second.

3.  Open the tablet controller:

https://<relay-ip>:8443/?mode=controller

Accept the SSL warning. Verify "controller connected" in the relay console. The full desktop UI appears plus an IGLOO Camera Control pad at the bottom.

4.  Test: Move a slider on the tablet. The IGLOO renderer should update immediately.

5.  Route the panoramic output to the IGLOO Core Engine.

## 6.2 Desktop Standalone Testing

To test without the relay server, open beltinator_vr.html directly in Chrome (no ?mode parameter). Use "Ricky’s Igloo Controller" panel in the right sidebar:

1.  Click the header bar to expand the panel.

2.  Select a resolution from the dropdown (default: 4000×2000).

3.  Click "Open Panorama" — a popup window opens showing the equirectangular panorama.

4.  The main desktop view is unaffected — panoramic rendering runs as a background GPU pass.

5.  Click "Close Panorama" to stop.

IMPORTANT: The popup must open as the first action from a user click. If the browser blocks it, allow popups for the site and try again.

## 6.3 Shutdown

Close browser tabs on IGLOO PC and tablet. Press Ctrl+C in the relay terminal.

# 7. Panoramic Output Specifications

## 7.1 Resolution Options

| Preset | Panorama | Cubemap Face | GPU Load |
| --- | --- | --- | --- |
| Light | 2000×1000 | 512×512 | Low |
| Standard (default) | 4000×2000 | 1024×1024 | Medium |
| High | 6000×3000 | 1536×1536 | High |
| Ultra | 8000×4000 | 2048×2048 | Very High |

Resolution can be changed live via the dropdown while panoramic mode is active.

## 7.2 Output Format

The output is a standard equirectangular panoramic image with 2:1 aspect ratio, centered on the camera position, covering the full 360° horizontal and 180° vertical field of view. This is the native format consumed by the IGLOO Core Engine for reprojection onto its six surfaces.

## 7.3 Performance

| GPU | 4000×2000 FPS | 8000×4000 FPS |
| --- | --- | --- |
| RTX 3070 | 45–60 fps | 20–30 fps |
| RTX 3090 / 4080 | 60 fps | 35–45 fps |
| RTX 4090 | 60+ fps | 50–60 fps |

## 7.4 Background Rendering

Browsers normally throttle rendering when a tab loses focus. The Beltinator includes a Web Worker-based timer that forces the animation loop to continue at ~60fps even when the browser tab is hidden. This ensures the panoramic output stream is never interrupted when the operator interacts with the output window, the IGLOO Core Engine interface, or other applications.

# 8. IGLOO Core Engine Integration

The IGLOO Core Engine captures the panoramic output window as a video source. The equirectangular image is reprojected onto the six projection surfaces by the IGLOO’s built-in cubemap/equirectangular reprojection pipeline.

Capture Methods

•  NDI Scan Converter: Point at the panoramic output window. The IGLOO Core Engine receives the NDI stream as a selectable video source.

•  Screen Capture: The IGLOO Core Engine may support direct window capture as an input source.

•  Spout: If available, a Spout sender can bridge the WebGL output to the IGLOO’s Spout receiver.

NOTE: If NDI Studio Monitor shows the source but the image does not update, verify that the Web Worker background rendering is active (check browser console for "Background rendering enabled"). Also verify the panoramic mode is running by checking the output window is continuously updating.

# 9. Tablet Controller Features

The controller displays the full Beltinator desktop interface plus an IGLOO Camera Control pad:

•  Left panel: Physics sliders (L*, Energy, Pitch Angle, diffusion coefficients, loss timescale), geomagnetic sliders (Kp, Dst, Vsw), visualization toggles, storm presets, satellite legend

•  Right panel: Computed values, governing equations with tabs, Ricky’s Igloo Controller, physics notes

•  Camera pad: Touch-drag to orbit the IGLOO camera around Earth. Pinch to zoom (2–40 Earth radii). Mouse wheel zoom supported.

•  Satellite focus: Click any satellite in the legend to send a focus command to the IGLOO renderer.

# 10. Troubleshooting

| Issue | Resolution |
| --- | --- |
| Panorama popup blocked | Allow popups for the site in browser settings. The popup must be the first action in the click handler — this is enforced in the current code. |
| SSL certificate warning | Click Advanced → Proceed. One-time step per browser. |
| WebSocket won’t connect | Verify relay is running, port 8443 is open, all machines on same LAN. Check relay console for connection messages. |
| Panoramic output freezes | Check browser console for "Background rendering enabled" message. The Web Worker should keep rendering when the tab loses focus. If not, the Worker may have failed to initialize. |
| NDI source visible but not updating | Verify the panoramic output window canvas is updating (visually check it). If the canvas updates but NDI doesn’t, the issue is in NDI Scan Converter’s capture rate, not the Beltinator. |
| Tablet sliders don’t affect IGLOO | Check relay console: both "igloo connected" and "controller connected" must appear. Check /status endpoint for client counts. |
| Low frame rate | Reduce panoramic resolution via dropdown. Disable particles toggle on controller. Run Chrome in fullscreen (F11). |
| Preset buttons don’t work via relay | Presets send lowercase names (quiet, moderate, intense, recovery). Verify the relay is forwarding messages (check relay console). |

# 11. WebSocket Protocol Reference

| Direction | type | data |
| --- | --- | --- |
| C → I | state | { lstar, energy, alpha, kp, dst, vsw, dll, dpp, daa, dap, tau, showBelts, showOrbits, showField, showParticles, showLossCone, mixedTerms } |
| C → I | preset | "quiet" ¦ "moderate" ¦ "intense" ¦ "recovery" |
| C → I | camera | { camH: float, camV: float, camDist: float } |
| C → I | focusSat | "RBSP" ¦ "THEMIS-A" ¦ "GOES" ¦ "LEO" ¦ "Lomonosov" |
| C → I | zoomBack | (no data) |
| I → C | computed | { bounce, psd, flux, dll, kp, lmp, lcds, blc, dlc, status } |
| I → C | status | "igloo_connected" ¦ "controller_connected" ¦ "*_disconnected" |

C = Controller, I = IGLOO. All messages are JSON. The relay forwards all messages from one side to the other without modification.

Document prepared by Dr. Nate Dailey, The MITRE Corporation. For CoLab-specific integration support, contact the MITRE CoLab Immersion Lab team.

The MITRE Corporation  |  NOAA Space Weather Operations  |  SpaceSherpa IR&D  |  March 2026